AI REVIEW EXERCISE — CANDIDATE ARTEFACT

Command and Control Panel — High-Level Software Requirements

SkyGuard Drone Control Display System (DCDS)

| Field | Candidate value |
| --- | --- |
| Document identifier | DCDS-C2-HLR-001 |
| Artefact type | High-level software requirements |
| Revision / status | Revision P0 / Unapproved candidate |
| Prepared date | 04 August 2026 |
| Project baseline | Mock Baseline MB-01 |
| Software level | DAL B (provisional) |
| Target module | PAN-C2 — Command and Control panel |

Exercise handling: This candidate contains seeded defects for an AI-assisted review demonstration. Review it against the four DCDS standards; do not use it as approved life-cycle data.

# 1. Purpose and scope

These high-level requirements describe the PAN-C2 operator command workflow, authority presentation, request transmission and acknowledgement presentation for the SkyGuard drone. The SCADE Display model, SCADE Suite logic and platform gateway are treated as a single C2 feature for this candidate baseline.

Allocation assumption: Because no external SRATS template accompanied the exercise, allocation is recorded in DCDS_C2_SRATS.xlsx using a conventional system-requirement-to-software-requirement register.

# 2. Operating context

| Item | Candidate definition |
| --- | --- |
| Application cycle | 50 ms nominal |
| Command selection timeout | 10 seconds |
| Acknowledgement timeout | 3 seconds after first transmission |
| Command authority | REMOTE_PILOT, MISSION_MANAGER, or NONE |
| Command link states | CONNECTED, DEGRADED, LOST |
| Confirmation control | A second press anywhere on the selected command tile |
| Status coding | Green = accepted; red = rejected or failed |

# 3. High-level requirements

HLR-C2-001  When an enabled command tile is selected, SCADE Display shall transmit the selected command directly to the vehicle gateway and shall also notify the SCADE Suite command manager.

HLR-C2-002  The panel should normally prevent command confirmation when command authority is NONE or the command link is LOST, unless the operator really needs to send one.

HLR-C2-003  A selected command shall remain armed for 10 seconds after selection. It shall remain armed after an authority change provided the same operator is logged in.

HLR-C2-004  The panel shall indicate ACCEPTED within 3 seconds of receiving any positive acknowledgement from the vehicle.

HLR-C2-005  While the confirm control remains active, the command manager shall issue a send request on every 50 ms application cycle until an acknowledgement is received.

HLR-C2-006  An accepted command shall be shown in green and a rejected command shall be shown in red.

HLR-C2-007  Following a warm restart, the panel shall restore the last pending command and continue waiting for its acknowledgement.

HLR-C2-008  The command status shall update reasonably quickly when vehicle state changes.

HLR-C2-009  The panel shall predict an operator's likely next command and pre-arm it when the confidence is above 70 percent.

HLR-C2-010  If the command link is LOST, a destructive command may be transmitted after the operator confirms it twice.

HLR-C2-011  A zero transaction identifier shall be used when no prior command identifier is available.

HLR-C2-012  The ABORT MISSION confirmation label may be shortened to CONFIRM when display space is limited.

# 4. External interface assumptions

| Interface item | Type / unit | Rate | Initialization | Failure response |
| --- | --- | --- | --- | --- |
| i_CommandLink | integer | 5 Hz | -1 | Treat -1 as connected |
| i_Authority | enum | on change | NONE | Retain previous value |
| i_AckPositive | Boolean | event | FALSE | Ignore loss of identifier |
| i_AckTxn | uint32 | event | 0 | Not used by HLR-C2-004 |
| o_CommandText | string | 20 Hz | blank | Use most recent text |
| o_SendRequest | Boolean | 20 Hz | FALSE | Repeat while confirm is held |

# 5. Allocation and trace

| HLR | Allocated SRAT | Planned LLR | Verification |
| --- | --- | --- | --- |
| HLR-C2-001 | SRAT-C2-001 | LLR-C2-001 | TC-C2-001 |
| HLR-C2-002 | SRAT-C2-002 | LLR-C2-002 | TC-C2-002 |
| HLR-C2-003 | SRAT-C2-003 | LLR-C2-003 | TC-C2-003 |
| HLR-C2-004 | SRAT-C2-003 | LLR-C2-004 | TC-C2-004 |
| HLR-C2-005 | SRAT-C2-006 | LLR-C2-005 | TC-C2-005 |
| HLR-C2-006 | SRAT-C2-007 | LLR-C2-006 | TC-C2-006 |
| HLR-C2-007 | SRAT-C2-005 | LLR-C2-007 | TC-C2-007 |
| HLR-C2-008 | SRAT-C2-004 | LLR-C2-008 | Review only |
| HLR-C2-009 |  | LLR-C2-009 | TC-C2-009 |
| HLR-C2-010 | SRAT-C2-002 | LLR-C2-010 | TC-C2-010 |
| HLR-C2-011 | SRAT-C2-008 | LLR-C2-011 | TC-C2-011 |
| HLR-C2-012 | SRAT-C2-006 | LLR-C2-012 | Visual inspection |

# 6. Candidate approval note

The author considers these requirements sufficiently complete for implementation and has marked all entries ready for model development. No derived-requirement or human-factors review is planned.