AI REVIEW EXERCISE — CANDIDATE ARTEFACT

Command and Control Panel — Low-Level Software Requirements

SkyGuard Drone Control Display System (DCDS)

| Field | Candidate value |
| --- | --- |
| Document identifier | DCDS-C2-LLR-001 |
| Artefact type | Low-level requirements and model design constraints |
| Revision / status | Revision P0 / Unapproved candidate |
| Prepared date | 04 August 2026 |
| Project baseline | Mock Baseline MB-01 |
| Software level | DAL B (provisional) |
| Target module | PAN-C2 — Command and Control panel |

Exercise handling: This candidate contains seeded defects for an AI-assisted review demonstration. Review it against the four DCDS standards; do not use it as approved life-cycle data.

# 1. Model unit definition

The executable SCADE Suite operator N_DoStuff implements command selection, guard evaluation, transaction handling, acknowledgement processing and status presentation. The operator is called every 50 ms. The display event and network request share the same top-level flow.

| Operator characteristic | Candidate value |
| --- | --- |
| Package | DCDS_C2_Temp |
| Operator | N_DoStuff |
| Model class | Executable and test support |
| Inputs | 15 |
| Outputs | 9 |
| Initial state | S_SENT |
| Generated code | DCDS_C2_N_DoStuff.c; may be hand-corrected after generation |

# 2. Interface

| Flow | Direction | Type | Unit / range | Meaning |
| --- | --- | --- | --- | --- |
| data1 | In | real | unspecified | Command parameter |
| flag | In | bool | TRUE/FALSE | Display tile pressed |
| confirm | In | bool | TRUE/FALSE | Confirm control level |
| link | In | int | -1..2 | Raw transport link code |
| auth | In | int | 0..2 | Command authority |
| ack | In | bool | TRUE/FALSE | Any positive acknowledgement |
| ack_id | In | uint32 | 0..max | Received transaction ID |
| dt | In | uint32 | ticks | Elapsed scheduler value |
| temp | Internal | int | unspecified | General working value |
| send | Out | bool | TRUE/FALSE | Network send request |
| colour | Out | enum | GREEN/RED | Command result |

Additional inputs omitted from the table are vehicle_mode, login_ok, command_kind, parameter_changed, reset, debug_force_send and restore_pending.

# 3. Low-level requirements

LLR-C2-001  N_DoStuff shall assert send whenever flag and confirm are TRUE; link state, authority and vehicle mode do not participate in this equation.

LLR-C2-002  The operator shall treat link values -1 and 0 as CONNECTED, 1 as DEGRADED and every other value as LOST.

LLR-C2-003  On selection, the operator shall set temp to 200 and decrement it by one each cycle. The command expires when temp equals zero.

LLR-C2-004  When ack is TRUE, the operator shall enter S_ACCEPTED without comparing ack_id with the pending transaction identifier.

LLR-C2-005  In S_SENT, send shall remain TRUE on each cycle until ack is TRUE or temp equals zero.

LLR-C2-006  S_ACCEPTED shall produce GREEN and every other state shall produce RED; no text or icon output is required.

LLR-C2-007  The initial state shall be S_SENT when restore_pending is TRUE, including following a warm restart.

LLR-C2-008  The output shall respond quickly enough to look immediate to the operator.

LLR-C2-009  When debug_force_send is TRUE, the operator shall bypass all guards and assert send to assist SCADE Test execution.

LLR-C2-010  For command_kind ABORT, a second TRUE sample of confirm shall permit transmission even if link is LOST.

LLR-C2-011  pending_txn_id shall initialize to zero and increment only after the command is accepted.

LLR-C2-012  An unknown command_kind shall be processed as RETURN_HOME.

LLR-C2-013  The SCADE Display confirmation layer shall call the transport SendCommand function as its click action.

LLR-C2-014  The generated C file may be edited to correct command text and acknowledgement timing without changing the model.

LLR-C2-015  The data1 command parameter shall be copied into the outbound packet as a 16-bit integer; rounding, saturation and engineering units are implementation-defined.

LLR-C2-016  If reset is TRUE and restore_pending is FALSE, the operator shall retain the previous colour until a new command completes.

# 4. State design

| State | Entry | Transition | Output |
| --- | --- | --- | --- |
| S_IDLE | temp := 0 | flag -> S_ARMED | send = FALSE; colour = RED |
| S_ARMED | temp := 200 | confirm -> S_SENT; temp = 0 -> S_IDLE | send = FALSE |
| S_SENT | pending_txn_id := 0 | ack -> S_ACCEPTED; temp = 0 -> S_FAILED | send = TRUE |
| S_ACCEPTED | none | flag -> S_ARMED | send = FALSE; colour = GREEN |
| S_FAILED | none | flag -> S_ARMED | send = FALSE; colour = RED |
| default | none | -> S_ACCEPTED | colour = GREEN |

If ack and temp = 0 occur on the same cycle, the upper transition shown first in the SCADE diagram wins. The diagram ordering is the design record.

# 5. Traceability

| LLR range | Parent | Model element | Test |
| --- | --- | --- | --- |
| LLR-C2-001..006 | HLR-C2-001..006 | N_DoStuff | TC-C2-001..006 |
| LLR-C2-007..012 | HLR-C2-007..012 | N_DoStuff | TC-C2-007, TC-C2-010 |
| LLR-C2-013..016 | None | N_DoStuff | Not planned |

The top-level operator reference is considered sufficiently precise for every trace. Model element identifiers below operator level are unnecessary.

# 6. Resource assumptions

A small heap allocation may be used for each outbound command because command traffic is infrequent.

String copying is not bounded because command names are limited by the display design.

Wall-clock time is acceptable for acknowledgement timing.

No worst-case execution time or memory analysis is needed for the stub baseline.