PROJECT SOFTWARE STANDARD

Software Design Standard

SkyGuard Drone Control Display System (DCDS)

| Field | Controlled value |
| --- | --- |
| Document identifier | DCDS-SDS-001 |
| Revision / status | Revision A / Mock approved baseline |
| Effective date | 04 August 2026 |
| Project baseline | Mock Baseline MB-01 |
| Software level | DAL B (provisional; subject to system safety assessment) |
| Toolchain | Ansys SCADE Suite, SCADE Display and SCADE Test - 2026 R1 project baseline |
| Owner | DCDS Software Engineering Authority |

Purpose: Defines architecture and detailed-design conventions for the four-panel DCDS application, including partitioning, interfaces, state behaviour, display safety and design traceability.

Use limitation: Mock standard for training and tailoring; not an approved certification or tool-qualification record. Replace mock approvals, tool versions and system assumptions before live use.

## Approval record

| Role | Mock approver | Disposition | Date |
| --- | --- | --- | --- |
| Software Engineering Authority | A. Engineer | Approved | 04 August 2026 |
| Software Verification Lead | V. Reviewer | Approved | 04 August 2026 |
| Software Quality Assurance | Q. Assessor | Approved | 04 August 2026 |

# 1. Scope and application

This standard applies to the DCDS application software, SCADE models, generated C source, manually developed integration code, configuration data and associated life-cycle data used to present drone state and initiate operator commands. Compliance is mandatory for software placed under the MB-01 certification baseline unless an approved deviation is recorded.

Assurance boundary: The mock baseline assigns DAL B because misleading awareness or loss of command presentation may contribute to a hazardous condition. The command transmission gateway independently validates every request. If that independence is not established, or if an erroneous command could directly cause a catastrophic condition, DAL A objectives and additional independence apply.

## 1.1 System overview

| Panel | Primary content | Safety-related behaviour |
| --- | --- | --- |
| Position (PAN-POS) | Latitude/longitude, map position, altitude, track, groundspeed, home bearing | Reject implausible data; show INVALID or STALE within 0.5 s; never dead-reckon without an explicit DR label |
| Communications (PAN-COM) | C2 link state, signal margin, latency, authentication, active bearer | Annunciate degraded and lost link; distinguish telemetry loss from command-link loss |
| Command and Control (PAN-C2) | Mode, authority, arm state, mission state, pending command, acknowledgement | Two-step guarded commands; inhibit on lost authority, invalid state or stale link; unique transaction ID |
| HUD (PAN-HUD) | Video/sensor view with heading, pitch, roll, altitude, speed and target cue overlay | Remove or flag invalid symbology; prevent stale overlay from appearing current; colour is never the sole cue |

## 1.2 Execution baseline

A deterministic 50 ms application cycle (20 Hz) is the logical execution rate.

Position data is expected at 10 Hz; communications health at 5 Hz; command status and core flight data at 20 Hz.

The video stream may run at 30 Hz but is not safety-credited. SCADE Display symbology is independently refreshed and validity-controlled.

All externally received values carry validity, source time and sequence information. The data-age manager produces CURRENT, DEGRADED, STALE or INVALID status.

Target code is generated as C and integrated with the project platform adaptation layer. Dynamic memory allocation is prohibited after initialization.

# 2. Design principles

SDS-PRN-001  The design shall make safety-significant behaviour explicit, deterministic and traceable. A reviewer shall be able to identify inputs, state, outputs, timing and failure response without inferring hidden behaviour.

SDS-PRN-002  Acquisition, validation, decision logic and presentation shall be separated. A panel shall not interpret an unvalidated transport value directly.

SDS-PRN-003  Common services such as age calculation, unit conversion, alert priority and command transaction handling shall be designed once and reused through controlled operators.

SDS-PRN-004  The design shall prefer simple data flow over unnecessary state. State machines are used only for genuinely modal or sequential behaviour.

SDS-PRN-005  Each safety-related output shall have a defined safe value or suppression behaviour for initialization, invalid input, stale input and internal fault.

SDS-PRN-006  Robustness shall be designed at the boundary; impossible values, stale frames, missing acknowledgements and repeated commands are expected conditions, not test-only anomalies.

# 3. Software architecture

| Layer | SCADE allocation | Responsibilities | Prohibited coupling |
| --- | --- | --- | --- |
| Input adaptation | Suite package DCDS_Inputs plus manual PAL | Decode, type conversion, timestamps, input snapshot | No panel-specific alert decisions |
| Validity services | Suite package DCDS_Validity | Range, age, sequence, plausibility and source status | No graphical object access |
| Application logic | Suite packages POS, COM, C2, HUD | Derived values, alert conditions, command gating, panel view models | No device-driver or rendering API calls |
| Presentation | Display project DCDS_HMI | Layering, symbols, text formatting, visibility and interaction bindings | No direct transport data or command transmission |
| Platform services | Manual PAL | Schedule, I/O, graphics, time, built-in test and target startup | No reimplementation of modelled functional requirements |

## 3.1 Data flow

At the start of each 50 ms cycle, the platform layer publishes an immutable input snapshot. DCDS_Validity evaluates range, sequence and age. Panel logic consumes only validated records and produces presentation-neutral view models. SCADE Display binds to those view models. Operator interactions produce requests that return to DCDS_C2, where authority, state, timeout and acknowledgement rules are applied before the platform gateway is called.

Key containment rule: SCADE Display is not allowed to transmit a command directly. A visual control may only emit a typed interaction event. DCDS_C2 converts an eligible event into a uniquely identified command request after all guards pass.

# 4. Interface design

| Interface | Producer -> consumer | Rate / timeout | Design contract |
| --- | --- | --- | --- |
| IF-NAV-01 | Navigation receiver -> DCDS_Validity | 10 Hz / stale at 500 ms | Position, altitude, track, speed, source time, sequence and integrity |
| IF-LINK-01 | Link manager -> DCDS_Validity | 5 Hz / degraded at 600 ms; lost at 1500 ms | Separate command/telemetry state, bearer, latency, margin and authentication |
| IF-CMD-01 | PAN-C2 -> command gateway | Event-driven / confirm within 5 s | Command type, parameters, authority token and unique transaction ID |
| IF-ACK-01 | Command gateway -> PAN-C2 | Event-driven / ack timeout 2 s | Transaction ID, accepted/rejected reason, vehicle state and source time |
| IF-HUD-01 | Flight data -> PAN-HUD | 20 Hz / stale at 250 ms | Attitude, heading, speed, altitude, target cue plus validity and age |

SDS-IF-001  Every interface item shall define name, type, unit, valid range, resolution, rate, latency, source, validity, initialization and failure response.

SDS-IF-002  Interface direction shall be unambiguous. Bidirectional state sharing through common globals is prohibited.

SDS-IF-003  Timing thresholds shall be requirements-derived and shall include tolerances that avoid boundary chatter.

SDS-IF-004  Compatibility changes to an interface shall be versioned and assessed for all producers, consumers, tests and target adapters.

# 5. Panel design rules

## 5.1 Position panel

The moving vehicle symbol, numeric coordinates and home vector use one validated navigation record.

CURRENT data is normally presented; DEGRADED data carries an amber border and source label; STALE or INVALID removes the vehicle symbol and replaces values with dashes.

Map pan/zoom is operator convenience only and shall not alter received coordinates or navigation state.

Track-up rotation is inhibited when track is invalid or groundspeed is below the defined reliability threshold; north-up remains available.

## 5.2 Communications panel

Command link, telemetry link and payload link are distinct channels with independent state and age.

Overall LINK LOST is asserted when the command link is lost; telemetry-only loss is labelled TELEMETRY LOST.

A transition from degraded to healthy requires three consecutive valid samples to prevent nuisance toggling.

Authentication failure has higher alert priority than low signal margin and is never collapsed into a generic disconnected state.

## 5.3 Command and Control panel

| State | Entry condition | Allowed operator action | Exit |
| --- | --- | --- | --- |
| IDLE | No command transaction | Select eligible command | Selection -> ARMED |
| ARMED | Command selected and guards valid | Confirm or cancel | Confirm -> SENT; timeout/guard loss -> IDLE |
| SENT | Gateway accepted request for transmission | Cancel only if command type supports cancellation | Ack -> ACCEPTED/REJECTED; 2 s -> TIMEOUT |
| ACCEPTED | Matching positive acknowledgement | Acknowledge message | Operator acknowledgement -> IDLE |
| REJECTED/TIMEOUT | Negative or missing acknowledgement | Acknowledge and review reason | Operator acknowledgement -> IDLE |

SDS-C2-001  A command selection shall expire after 5 seconds and on authority, vehicle mode, link state or parameter change.

SDS-C2-002  A confirmation shall generate one transaction identifier and at most one send request. Repeated button activation shall not duplicate the request.

SDS-C2-003  The displayed acknowledgement shall match the pending transaction identifier. Late or mismatched acknowledgements are logged but shall not indicate command success.

SDS-C2-004  Destructive commands such as ABORT MISSION or RETURN HOME shall use a spatially separated confirm control and explicit command text.

## 5.4 HUD panel

Video and safety-related symbology are separate layers. The video may freeze without freezing the overlay validity timer.

Attitude cues are removed within 250 ms of invalid or stale attitude data and replaced with ATTITUDE INVALID.

Heading tape wraps at 359.99 to 0.00 degrees without discontinuity; numeric heading is three digits.

Target cues outside the field of view are clamped to an edge indicator with direction; they are never drawn at an invented on-screen position.

# 6. Alert and HMI design

| Priority | Presentation | Acknowledgement | Examples |
| --- | --- | --- | --- |
| Warning | Red, fixed location, text plus icon; aural request to platform | Latched until condition clears and operator acknowledges | COMMAND LINK LOST; UNAUTHORISED CONTROL |
| Caution | Amber, text plus icon | Latched until condition clears or is acknowledged as specified | POSITION DEGRADED; HIGH LATENCY |
| Advisory | Cyan/white information banner | Not normally latched | PAYLOAD LINK SWITCHED; MAP NORTH-UP |

SDS-HMI-001  Alert priority, wording, colour, icon and latching shall be produced from one controlled alert catalogue.

SDS-HMI-002  Normal, degraded, stale and invalid states shall be visually distinguishable under day and night palettes and common colour-vision deficiencies.

SDS-HMI-003  A safety-significant value shall include its unit or an unambiguous fixed unit label.

SDS-HMI-004  Touch/cursor targets shall not overlap. Command controls shall be disabled visually and logically when guards fail.

SDS-HMI-005  Display refresh failure shall be detectable by a platform heartbeat or watchdog independent of the rendered content.

# 7. Requirements and design traceability

| High-level requirement | Design element | Low-level/design rule | Verification |
| --- | --- | --- | --- |
| HLR-POS-004 Show stale navigation data | DCDS_Validity.NavAge + PAN-POS view model | LLR-POS-014 stale above 500 ms; SDS-POS behaviour | TC-POS-002; review DR-POS-07 |
| HLR-COM-006 Annunciate command-link loss | DCDS_COM.LinkState | LLR-COM-021 lost at 1500 ms | TC-COM-004; image IMG-COM-004 |
| HLR-C2-011 Prevent unintended command | DCDS_C2.CommandTxn | LLR-C2-031 two-step, expiry and single-send | TC-C2-006/007; MC/DC set C2-MCDC-01 |
| HLR-HUD-009 Remove invalid attitude | DCDS_HUD.Validity + Display HUD_Attitude layer | LLR-HUD-018 hide at age >250 ms | TC-HUD-005; image IMG-HUD-005 |

SDS-TRC-001  Traceability shall be bidirectional from system-derived software requirements through design/model elements, generated or manual implementation, verification cases and results.

SDS-TRC-002  A derived requirement shall be identified, justified and fed back to the system safety and requirements processes before approval.

SDS-TRC-003  One trace link shall identify the exact model element or interface, not only the top-level package.

# 8. Design review and acceptance

Confirm the allocated requirements and safety constraints are current and complete.

Review architecture, interfaces, timing, state, initialization, data/control coupling and safe responses.

Inspect each panel in normal, degraded, stale, invalid and restart conditions using model simulation.

Confirm derived requirements, assumptions and open decisions are visible and owned.

Record review participants, independence, findings, dispositions and baseline identifiers.

# Appendix A - Design review checklist

| Review question | Expected evidence | Result |
| --- | --- | --- |
| Are all inputs typed, ranged, timed and validity-controlled? | Interface control data and model ports | Pass / Fail / N/A |
| Are initialization and all modal transitions explicit? | State transition review and restart simulation | Pass / Fail / N/A |
| Can any display element bypass validated data? | Binding and data-flow review | Pass / Fail / N/A |
| Can any UI event bypass command guards? | Interaction trace and C2 transaction model | Pass / Fail / N/A |
| Are stale/invalid states unmistakable without colour alone? | Display simulation and image baseline | Pass / Fail / N/A |
| Are safety, timing and resource assumptions verified or constrained? | Assumption log and linked verification | Pass / Fail / N/A |

## Conformance and deviations

A work product conforms when every applicable shall statement is met and the review evidence is recorded. A deviation shall identify the affected rule, technical rationale, safety impact, compensating verification, owner and approval. Software Quality Assurance records deviations and confirms closure before release.

# References

| Reference | Use in this standard |
| --- | --- |
| RTCA DO-178C | Software Considerations in Airborne Systems and Equipment Certification, 13 December 2011. |
| RTCA DO-330 | Software Tool Qualification Considerations, 13 December 2011. |
| RTCA DO-331 | Model-Based Development and Verification Supplement to DO-178C and DO-278A, 13 December 2011. |
| FAA AC 20-115D | Airborne Software Development Assurance Using EUROCAE ED-12() and RTCA DO-178(). |
| FAA Order 8110.49A | Software Approval Guidelines. |
| Ansys SCADE Suite | Official product information for model-based development and qualifiable code generation. |
| Ansys SCADE Display | Official product information for safety-critical HMI development and code generation. |

FAA AC 20-115D | FAA Order 8110.49A | Ansys SCADE Suite | Ansys SCADE Display

Project references: DCDS-PSAC-001 Plan for Software Aspects of Certification; DCDS-SDP-001 Software Development Plan; DCDS-SVP-001 Software Verification Plan; DCDS-SCMP-001 Software Configuration Management Plan; DCDS-SQAP-001 Software Quality Assurance Plan; DCDS-SCI-001 Software Configuration Index. These are mock companion records and are not included in this package.