PROJECT SOFTWARE STANDARD

Software Implementation Standard

SkyGuard Drone Control Display System (DCDS)

| Field | Controlled value |
| --- | --- |
| Document identifier | DCDS-SIS-001 |
| Revision / status | Revision A / Mock approved baseline |
| Effective date | 04 August 2026 |
| Project baseline | Mock Baseline MB-01 |
| Software level | DAL B (provisional; subject to system safety assessment) |
| Toolchain | Ansys SCADE Suite, SCADE Display and SCADE Test - 2026 R1 project baseline |
| Owner | DCDS Software Engineering Authority |

Purpose: Defines mandatory implementation rules for SCADE-generated application code, manually developed platform/integration code, build configuration and target integration for the four-panel DCDS application.

Use limitation: Mock standard for training and tailoring; not an approved certification or tool-qualification record. Replace mock approvals, tool versions and system assumptions before live use.

## Approval record

| Role | Mock approver | Disposition | Date |
| --- | --- | --- | --- |
| Software Engineering Authority | A. Engineer | Approved | 04 August 2026 |
| Software Verification Lead | V. Reviewer | Approved | 04 August 2026 |
| Software Quality Assurance | Q. Assessor | Approved | 04 August 2026 |

# 1. Scope and application

This standard applies to the DCDS application software, SCADE models, generated C source, manually developed integration code, configuration data and associated life-cycle data used to present drone state and initiate operator commands. Compliance is mandatory for software placed under the MB-01 certification baseline unless an approved deviation is recorded.

Assurance boundary: The mock baseline assigns DAL B because misleading awareness or loss of command presentation may contribute to a hazardous condition. The command transmission gateway independently validates every request. If that independence is not established, or if an erroneous command could directly cause a catastrophic condition, DAL A objectives and additional independence apply.

## 1.1 System overview

| Panel | Primary content | Safety-related behaviour |
| --- | --- | --- |
| Position (PAN-POS) | Latitude/longitude, map position, altitude, track, groundspeed, home bearing | Reject implausible data; show INVALID or STALE within 0.5 s; never dead-reckon without an explicit DR label |
| Communications (PAN-COM) | C2 link state, signal margin, latency, authentication, active bearer | Annunciate degraded and lost link; distinguish telemetry loss from command-link loss |
| Command and Control (PAN-C2) | Mode, authority, arm state, mission state, pending command, acknowledgement | Two-step guarded commands; inhibit on lost authority, invalid state or stale link; unique transaction ID |
| HUD (PAN-HUD) | Video/sensor view with heading, pitch, roll, altitude, speed and target cue overlay | Remove or flag invalid symbology; prevent stale overlay from appearing current; colour is never the sole cue |

## 1.2 Execution baseline

A deterministic 50 ms application cycle (20 Hz) is the logical execution rate.

Position data is expected at 10 Hz; communications health at 5 Hz; command status and core flight data at 20 Hz.

The video stream may run at 30 Hz but is not safety-credited. SCADE Display symbology is independently refreshed and validity-controlled.

All externally received values carry validity, source time and sequence information. The data-age manager produces CURRENT, DEGRADED, STALE or INVALID status.

Target code is generated as C and integrated with the project platform adaptation layer. Dynamic memory allocation is prohibited after initialization.

# 2. Implementation strategy

The implementation is split between generated functional code and a small, isolated platform adaptation layer. SCADE Suite contains data validation, state management, command gating and panel view-model logic. SCADE Display contains graphical composition and bindings. Hand code is limited to device drivers, operating-system adaptation, transport decoding, graphics/platform startup and generated-code scheduling.

| Implementation unit | Technology | Permitted responsibility | Primary evidence |
| --- | --- | --- | --- |
| DCDS_Core | SCADE Suite | Input validation, freshness, modes, alerts, command request state machine, display values | Model baseline, generated code log, review and tests |
| DCDS_HMI | SCADE Display | Panel layout, layers, symbology, textual/graphical annunciation and bindings | Display model, rule-check report, image tests |
| DCDS_PAL | Manual C | I/O conversion, scheduler call, time service, driver and graphics interface | Reviewed source, static analysis, object tests |
| DCDS_Test | SCADE Test | Executable requirements-based scenarios, procedures, expected values and images | Test project, procedures, results and review record |

# 3. General implementation rules

SIS-IMP-001  Implementation shall be traceable to approved low-level requirements and design data. Derived implementation behaviour is prohibited; any discovered need shall be returned to design and requirements control.

SIS-IMP-002  Each executable unit shall have one defined owner, purpose, interface and cycle rate. Unused outputs and dead logic shall be removed or justified.

SIS-IMP-003  All externally sourced values shall be range-checked, validity-checked and age-checked before use by safety-related logic or display bindings.

SIS-IMP-004  Initial values shall represent a known safe state: data INVALID, command authority NONE, no command pending, alert lamps tested then deasserted, and HUD safety symbology hidden until valid.

SIS-IMP-005  The implementation shall be deterministic for a given initial state and ordered input sequence. Hidden dependence on wall-clock time, uninitialised storage or iteration order is prohibited.

SIS-IMP-006  Resource use shall be bounded. Recursion, unbounded loops, post-initialisation heap allocation, uncontrolled file access and blocking I/O in the 50 ms application cycle are prohibited.

SIS-IMP-007  Numerical conversions shall be explicit. Range, resolution, unit and overflow behaviour shall be specified at each external interface.

SIS-IMP-008  A safety-significant status shall never be represented by colour alone. Text, shape, location or state iconography shall provide a second cue.

# 4. Data and interface implementation

| Type | Representation | Valid range / resolution | Invalid handling |
| --- | --- | --- | --- |
| T_Latitude_deg_e7 | signed 32-bit scaled integer | -900000000 to +900000000; 1e-7 degree | Set position invalid; preserve raw value only in diagnostic channel |
| T_Longitude_deg_e7 | signed 32-bit scaled integer | -1800000000 to +1800000000; 1e-7 degree | Set position invalid |
| T_Heading_deg_e2 | unsigned 16-bit scaled integer | 0 to 35999; 0.01 degree | Hide heading cue and show HDG INVALID |
| T_Altitude_m_e1 | signed 32-bit scaled integer | -1000.0 m to 50000.0 m; 0.1 m | Display dashes with ALT INVALID |
| T_DataAge_ms | unsigned 32-bit integer | 0 to 600000 ms | Saturate at maximum; STALE above source-specific threshold |
| T_CommandId | unsigned 32-bit integer | 1 to 4294967295; zero reserved | Reject zero and duplicate identifiers |

SIS-DAT-001  Physical units shall appear in type names, interface definitions and displayed labels. A unit conversion shall occur once at an identified boundary.

SIS-DAT-002  Validity and age shall be transported separately from the engineering value; sentinel numeric values shall not encode validity.

SIS-DAT-003  Sequence counters shall use modulo arithmetic with an explicit rollover rule. Duplicate and out-of-order data shall not refresh the age timer.

SIS-DAT-004  Shared data exchanged with the platform layer shall use project-owned fixed-width types and shall not expose compiler-dependent enum width, bit-field layout or structure padding.

# 5. SCADE generated-code controls

SIS-GEN-001  SCADE Suite and SCADE Display code generation shall use a configuration identified in DCDS-SCI-001. Generator options, target settings, libraries and patches shall be configuration-controlled.

SIS-GEN-002  Generated source shall not be manually edited. A required correction shall be made in the approved model, generation configuration or isolated wrapper.

SIS-GEN-003  Qualification credit shall be claimed only for the exact tool version, qualified feature set, target language and operational constraints covered by approved qualification data.

SIS-GEN-004  Every generation run shall retain the model identifier, configuration checksum, generator identity, command/options, warnings, generated source checksum and completion status.

SIS-GEN-005  Generator warnings shall be zero or individually dispositioned before source compilation. Suppression of a warning requires approval and objective evidence that the suppression is safe.

SIS-GEN-006  Hand-written wrappers shall not alter generated internal state, bypass input validation or call generated step functions more than once per scheduled cycle.

# 6. Manual C and platform adaptation layer

| Area | Required rule | Example |
| --- | --- | --- |
| Language subset | ISO C99 project subset; compiler extensions disabled unless approved | stdint.h fixed-width types; no variable-length arrays |
| Control flow | Single-entry/single-exit preferred; bounded loops; every switch has explicit default | Frame-copy loop bounded by DCDS_MAX_SIGNALS |
| Memory | Static allocation after startup; array bounds demonstrated | Preallocated I/O buffers and testable length checks |
| Concurrency | Only the scheduler task invokes model step functions; I/O copied through an atomic snapshot | Receive task publishes complete frame with sequence counter |
| Errors | Return status is checked and mapped to an identified safe response | Graphics init failure prevents operational mode |
| Diagnostics | Diagnostic logging shall not change control timing or safety state | Non-blocking ring buffer; overflow counter |

SIS-C-001  All functions shall have prototypes and internal linkage unless they form an approved interface. Implicit declarations and implicit narrowing conversions are prohibited.

SIS-C-002  Pointers shall be non-null before dereference. Pointer arithmetic is restricted to bounded buffers within the platform adaptation layer.

SIS-C-003  A compiler or linker warning shall be treated as an error unless an approved, reviewed exception is recorded.

SIS-C-004  Compiler optimisation settings shall be fixed. Object-code verification shall address any source-to-object behaviour not covered by qualified generation or source-based evidence.

# 7. Panel implementation rules

| Panel | Implementation rule | Safe response |
| --- | --- | --- |
| PAN-POS | Map and numeric position use the same validated data record and age state | Remove moving symbol; retain map; show POSITION STALE/INVALID banner |
| PAN-COM | Command and telemetry links are evaluated independently | Show exact failed channel and start loss timer; do not show green CONNECTED |
| PAN-C2 | Command is armed by selection and sent only after confirmation while authority remains valid | Cancel pending request on timeout, authority change, mode change or link loss |
| PAN-HUD | Every safety-related overlay binding includes value, validity and age | Hide invalid overlay element and show labelled cue; video alone is not treated as valid navigation data |

# 8. Build, integration and release

Restore the approved model, display asset, wrapper source, compiler and generator baselines from configuration control.

Run model and display rule checks; resolve or approve every finding.

Generate Suite and Display source; archive logs and checksums.

Compile with the controlled compiler response file; fail the build on warnings.

Run static analysis, unit/model tests and target integration tests; retain machine-readable and reviewed reports.

Produce the executable, load image, map file and build manifest; verify checksums and sign the Software Configuration Index entry.

| Release gate | Acceptance evidence |
| --- | --- |
| Traceability | All implemented units trace bidirectionally to approved requirements/design and tests |
| Analysis | No open high-severity rule-check, compiler, static-analysis or data-coupling finding |
| Testing | All required cases passed on the release baseline; failures resolved or formally accepted |
| Configuration | SCI identifies source, models, assets, tools, options, executable and qualification data |
| Quality | SQA audit confirms standards compliance and approved deviations |

# Appendix A - Implementation review checklist

| Check | Reviewer evidence | Result |
| --- | --- | --- |
| Inputs checked for range, validity, age and sequence | Trace to interface definition and robustness tests | Pass / Fail / N/A |
| Initial and restart states are explicit | Model initialization review and TC-SYS-010 | Pass / Fail / N/A |
| Generated code is unmodified and reproducible | Checksum comparison and generation log | Pass / Fail / N/A |
| Manual code meets bounded memory/control-flow rules | Review record and static-analysis report | Pass / Fail / N/A |
| No colour-only alert or stale-current ambiguity | Panel review and image-comparison tests | Pass / Fail / N/A |
| Target build is warning-free and baseline identified | Build manifest and SCI | Pass / Fail / N/A |

## Conformance and deviations

A work product conforms when every applicable shall statement is met and the review evidence is recorded. A deviation shall identify the affected rule, technical rationale, safety impact, compensating verification, owner and approval. Software Quality Assurance records deviations and confirms closure before release.

# References

| Reference | Use in this standard |
| --- | --- |
| RTCA DO-178C | Software Considerations in Airborne Systems and Equipment Certification, 13 December 2011. |
| RTCA DO-330 | Software Tool Qualification Considerations, 13 December 2011. |
| RTCA DO-331 | Model-Based Development and Verification Supplement to DO-178C and DO-278A, 13 December 2011. |
| FAA AC 20-115D | Airborne Software Development Assurance Using EUROCAE ED-12() and RTCA DO-178(). |
| FAA Order 8110.49A | Software Approval Guidelines. |
| Ansys SCADE Suite | Official product information for model-based development and qualifiable code generation. |
| Ansys SCADE Display | Official product information for safety-critical HMI development and code generation. |

FAA AC 20-115D | FAA Order 8110.49A | Ansys SCADE Suite | Ansys SCADE Display

Project references: DCDS-PSAC-001 Plan for Software Aspects of Certification; DCDS-SDP-001 Software Development Plan; DCDS-SVP-001 Software Verification Plan; DCDS-SCMP-001 Software Configuration Management Plan; DCDS-SQAP-001 Software Quality Assurance Plan; DCDS-SCI-001 Software Configuration Index. These are mock companion records and are not included in this package.