PROJECT SOFTWARE STANDARD

Software Modelling Standard

SkyGuard Drone Control Display System (DCDS)

| Field | Controlled value |
| --- | --- |
| Document identifier | DCDS-SMS-001 |
| Revision / status | Revision A / Mock approved baseline |
| Effective date | 04 August 2026 |
| Project baseline | Mock Baseline MB-01 |
| Software level | DAL B (provisional; subject to system safety assessment) |
| Toolchain | Ansys SCADE Suite, SCADE Display and SCADE Test - 2026 R1 project baseline |
| Owner | DCDS Software Engineering Authority |

Purpose: Defines DO-331-aligned modelling rules for SCADE Suite functional models and SCADE Display graphical models used as design, low-level requirements and generated-code inputs for DCDS.

Use limitation: Mock standard for training and tailoring; not an approved certification or tool-qualification record. Replace mock approvals, tool versions and system assumptions before live use.

## Approval record

| Role | Mock approver | Disposition | Date |
| --- | --- | --- | --- |
| Software Engineering Authority | A. Engineer | Approved | 04 August 2026 |
| Software Verification Lead | V. Reviewer | Approved | 04 August 2026 |
| Software Quality Assurance | Q. Assessor | Approved | 04 August 2026 |

# 1. Scope and application

This standard applies to the DCDS application software, SCADE models, generated C source, manually developed integration code, configuration data and associated life-cycle data used to present drone state and initiate operator commands. Compliance is mandatory for software placed under the MB-01 certification baseline unless an approved deviation is recorded.

Assurance boundary: The mock baseline assigns DAL B because misleading awareness or loss of command presentation may contribute to a hazardous condition. The command transmission gateway independently validates every request. If that independence is not established, or if an erroneous command could directly cause a catastrophic condition, DAL A objectives and additional independence apply.

## 1.1 System overview

| Panel | Primary content | Safety-related behaviour |
| --- | --- | --- |
| Position (PAN-POS) | Latitude/longitude, map position, altitude, track, groundspeed, home bearing | Reject implausible data; show INVALID or STALE within 0.5 s; never dead-reckon without an explicit DR label |
| Communications (PAN-COM) | C2 link state, signal margin, latency, authentication, active bearer | Annunciate degraded and lost link; distinguish telemetry loss from command-link loss |
| Command and Control (PAN-C2) | Mode, authority, arm state, mission state, pending command, acknowledgement | Two-step guarded commands; inhibit on lost authority, invalid state or stale link; unique transaction ID |
| HUD (PAN-HUD) | Video/sensor view with heading, pitch, roll, altitude, speed and target cue overlay | Remove or flag invalid symbology; prevent stale overlay from appearing current; colour is never the sole cue |

## 1.2 Execution baseline

A deterministic 50 ms application cycle (20 Hz) is the logical execution rate.

Position data is expected at 10 Hz; communications health at 5 Hz; command status and core flight data at 20 Hz.

The video stream may run at 30 Hz but is not safety-credited. SCADE Display symbology is independently refreshed and validity-controlled.

All externally received values carry validity, source time and sequence information. The data-age manager produces CURRENT, DEGRADED, STALE or INVALID status.

Target code is generated as C and integrated with the project platform adaptation layer. Dynamic memory allocation is prohibited after initialization.

# 2. Model role and classification

The project model is not an informal drawing. Approved SCADE Suite elements constitute detailed design and, where explicitly tagged, low-level requirements used to generate source code. SCADE Display elements constitute graphical design and detailed presentation requirements used to generate display code/assets. Higher-level requirements remain textual and trace to model elements. The model role is declared at package and operator level.

| Model class | Life-cycle role | Permitted content | Verification expectation |
| --- | --- | --- | --- |
| M-HLR | Executable elaboration of a high-level requirement | Unambiguous externally visible behaviour only | Requirements review plus independent tests |
| M-LLR | Low-level requirement and detailed design | Algorithms, state, boundary logic and exact interface behaviour | Model review, requirements tests and model coverage |
| M-DES | Architecture/design support | Decomposition, data flow, non-executable annotations | Design review and traceability |
| M-HMI | Graphical detailed design / generation input | Layers, bindings, assets, visual states and interactions | Rule checks, image tests and target display tests |
| M-TEST | Verification model or oracle | Test-only drivers, monitors and independent computations | Separate baseline, review and qualification determination |

SMS-CLS-001  Every package and operator shall declare its model class and linked requirements. Mixed requirement and test logic within one operator is prohibited.

SMS-CLS-002  A model used for code generation shall contain only approved executable behaviour, documented initialization and justified library instances.

SMS-CLS-003  Model-based credit shall be claimed only through the approved DO-331 approach and project plans; simulation alone does not establish compliance.

# 3. SCADE Suite structure and naming

| Element | Convention | Example |
| --- | --- | --- |
| Project/package | DCDS_<domain> | DCDS_Validity, DCDS_C2 |
| Operator/node | N_<verb>_<object> | N_ValidatePosition, N_ManageCommandTxn |
| Type | T_<noun>[_unit_scale] | T_Heading_deg_e2, T_LinkState |
| Constant | C_<noun>_<unit> | C_NAV_STALE_MS, C_CONFIRM_TIMEOUT_MS |
| Input/output/local | i_<noun>, o_<noun>, l_<noun> | i_navFrame, o_positionView, l_isFresh |
| State | S_<state> | S_IDLE, S_ARMED, S_SENT |
| Display object | <panel>_<role>_<noun> | POS_TXT_Altitude, C2_BTN_Confirm |
| Requirement tag | REQ:<identifier> | REQ:HLR-C2-011 |

SMS-NAM-001  Names shall convey domain meaning, unit and role. Generic names such as temp, data1, flag or block are prohibited in approved models.

SMS-NAM-002  Abbreviations shall come from the project glossary. Similar concepts shall not use competing abbreviations.

SMS-NAM-003  Operator interfaces shall remain stable and small; more than 12 inputs or 8 outputs requires decomposition or approved justification.

# 4. Data-flow modelling rules

SMS-DF-001  Each equation shall have one clear purpose. Complex expressions shall be decomposed into named intermediate flows that expose boundary and validity decisions.

SMS-DF-002  Units and ranges shall be represented through project types and annotations. Mixing quantities with different units in an expression is prohibited.

SMS-DF-003  Boolean logic with three or more independent conditions shall use named conditions and shall be formatted to make precedence visible.

SMS-DF-004  Numeric overflow, saturation, division by zero and conversion behaviour shall be explicit. Reliance on target-language wraparound is prohibited.

SMS-DF-005  Default branches shall produce a safe, defined result and shall be verified. A default branch shall not silently treat an unknown enum as NORMAL.

SMS-DF-006  Feedback shall use explicit delay/state operators with an initial value linked to a requirement. Combinational cycles are prohibited.

SMS-DF-007  Operators shall be deterministic and side-effect free except at the approved top-level platform boundary.

Preferred pattern: For an external value, model four named stages: IsRangeValid -> IsSequenceValid -> DataAge -> ValidatedRecord. The panel consumes the ValidatedRecord, never the raw frame.

# 5. State-machine rules

SMS-STM-001  A state machine shall model a true mode or sequence and shall have a textual state/transition table linked to the model.

SMS-STM-002  Initial state and entry actions shall be explicit. Initialization shall not emit an external command or present data as valid.

SMS-STM-003  Transitions from a state shall be mutually exclusive or have documented priority. Priority based only on diagram placement is prohibited.

SMS-STM-004  Every state shall define behaviour for timeout, invalid input and unexpected enum values.

SMS-STM-005  Transient events shall be sampled once and acknowledged or cleared deterministically to prevent repeated action across cycles.

SMS-STM-006  State count above eight, nesting deeper than two levels or cross-region dependencies require architecture review.

| C2 transition | Guard | Action | Forbidden alternative |
| --- | --- | --- | --- |
| IDLE -> ARMED | eligible command AND valid authority AND link available | Store immutable command parameters; start 5 s timer | Arming from a disabled visual state |
| ARMED -> SENT | confirm event AND timer valid AND guards still valid | Allocate transaction ID; emit one-cycle send pulse | Holding confirm produces repeated pulses |
| ARMED -> IDLE | cancel OR timeout OR guard loss | Clear parameters and pending visual state | Retaining an expired selection |
| SENT -> ACCEPTED | ack ID matches AND result accepted | Latch accepted status for operator acknowledgement | Any positive ack satisfies pending request |

# 6. Timing, clocks and initialization

SMS-TIM-001  The logical cycle is 50 ms. Timers shall count controlled elapsed milliseconds supplied by the platform and shall define behaviour for missed or repeated ticks.

SMS-TIM-002  Threshold comparison semantics shall be explicit: for example, age > 500 ms is STALE while age <= 500 ms remains CURRENT unless the requirement states otherwise.

SMS-TIM-003  Timer saturation shall be modelled to prevent rollover from making old data appear current.

SMS-TIM-004  Reset and warm-restart behaviour shall be modelled and tested. Restored persisted state shall not include command-pending or data-valid flags.

SMS-TIM-005  Rate-transition logic shall define hold, sample and invalidation behaviour; implicit assumptions about producer/consumer phase are prohibited.

# 7. SCADE Display modelling

| Topic | Mandatory standard |
| --- | --- |
| Coordinate system | Use the approved 1920 x 1080 logical canvas and panel-local containers; safety text remains within the defined safe area |
| Layering | Background < map/video < normal symbology < caution < warning < modal command confirmation |
| Bindings | Bind only to typed panel view-model outputs; no raw bus/transport fields |
| Visibility | Each safety item has explicit visible, degraded and invalid conditions; default is hidden for invalid data |
| Assets/fonts | Use only configuration-controlled assets and embedded font pack; scaling and fallback fonts prohibited |
| Interaction | UI objects emit typed events; functional command guards remain in SCADE Suite |
| Colour | Use controlled palette tokens; colour supplemented by text/icon/shape; no arbitrary per-object colours |

SMS-DSP-001  A graphical object shall have a unique name, panel owner, layer, requirement link and controlled z-order.

SMS-DSP-002  Text boxes shall be sized for the longest approved string plus 15 percent horizontal margin at the controlled font and resolution.

SMS-DSP-003  Scaling, rotation and clipping limits shall be specified. Out-of-range symbology shall clamp, hide or show an off-screen cue as required; it shall not wrap accidentally.

SMS-DSP-004  Warning and command-confirmation layers shall not be obscured by map, video, menus or advisory layers.

SMS-DSP-005  Animation used for emphasis shall have a deterministic period and phase source. Safety meaning shall not depend solely on animation.

SMS-DSP-006  Image baselines shall identify resolution, theme, font pack, graphics driver configuration and allowed comparison tolerance.

# 8. Model layout and readability

Data flows left to right and control/state progression top to bottom unless a local annotation explains another arrangement.

Crossings are minimised; long flows use named connections or encapsulated operators, not ambiguous visual wiring.

A diagram should fit on one review screen at normal zoom. Larger logic is decomposed by domain or transformation stage.

Comments explain why, assumptions and safety intent. They do not restate obvious equations.

Disabled, experimental or debug elements are removed before baseline; comments do not substitute for configuration control.

# 9. Libraries and reuse

SMS-LIB-001  Reusable operators and display widgets shall be configuration-controlled, versioned, documented and verified for their claimed domain and assumptions.

SMS-LIB-002  A library instance shall be traced to the requirements it satisfies and to the exact library version. Copy-and-modify reuse is prohibited.

SMS-LIB-003  Third-party or vendor model libraries shall be assessed for qualification/verification credit, configuration control, limitations and generated-code impact.

SMS-LIB-004  Project wrappers shall constrain a generic library operator to project types, ranges, timing and safe defaults.

# 10. Model verification and baseline

Run SCADE model checks and project custom rules with zero unexplained errors or warnings.

Review requirements links, classification, interfaces, initialization, state and safe responses.

Simulate normal, boundary, invalid, stale and restart scenarios before code generation.

Generate code only from an approved model baseline and retain model/generator checksums and logs.

Execute the SCADE Test campaign and analyse model/source coverage for unexercised or unintended elements.

| Prohibited pattern | Reason | Required alternative |
| --- | --- | --- |
| Magic numeric constants | Units and safety meaning are hidden | Named, typed constant linked to requirement |
| Raw value bound to display | Bypasses range/age/validity control | Validated panel view-model record |
| Unprioritised overlapping transitions | Execution depends on hidden priority | Mutually exclusive guards or documented priority |
| Edited generated code | Breaks model-code consistency and reproducibility | Change model/configuration or isolated wrapper |
| Colour-only alert | Meaning lost under display/vision conditions | Text/icon/shape plus controlled colour |
| Test logic inside production model | Contaminates executable baseline | Separate M-TEST package and harness |

# Appendix A - Model review checklist

| Check | Evidence | Result |
| --- | --- | --- |
| Model role/classification and requirement links are explicit | Package/operator properties and trace report | Pass / Fail / N/A |
| Types, units, ranges and conversions are controlled | Type package and interface review | Pass / Fail / N/A |
| State, timing, initialization and failure responses are complete | Model review and scenario results | Pass / Fail / N/A |
| Display bindings use validated view models | Binding report and panel inspection | Pass / Fail / N/A |
| No prohibited pattern or unexplained rule-check finding remains | Rule-check report and deviations | Pass / Fail / N/A |
| Generation is reproducible from identified inputs | Configuration, logs and checksum comparison | Pass / Fail / N/A |

## Conformance and deviations

A work product conforms when every applicable shall statement is met and the review evidence is recorded. A deviation shall identify the affected rule, technical rationale, safety impact, compensating verification, owner and approval. Software Quality Assurance records deviations and confirms closure before release.

# References

| Reference | Use in this standard |
| --- | --- |
| RTCA DO-178C | Software Considerations in Airborne Systems and Equipment Certification, 13 December 2011. |
| RTCA DO-330 | Software Tool Qualification Considerations, 13 December 2011. |
| RTCA DO-331 | Model-Based Development and Verification Supplement to DO-178C and DO-278A, 13 December 2011. |
| FAA AC 20-115D | Airborne Software Development Assurance Using EUROCAE ED-12() and RTCA DO-178(). |
| FAA Order 8110.49A | Software Approval Guidelines. |
| Ansys SCADE Suite | Official product information for model-based development and qualifiable code generation. |
| Ansys SCADE Display | Official product information for safety-critical HMI development and code generation. |

FAA AC 20-115D | FAA Order 8110.49A | Ansys SCADE Suite | Ansys SCADE Display

Project references: DCDS-PSAC-001 Plan for Software Aspects of Certification; DCDS-SDP-001 Software Development Plan; DCDS-SVP-001 Software Verification Plan; DCDS-SCMP-001 Software Configuration Management Plan; DCDS-SQAP-001 Software Quality Assurance Plan; DCDS-SCI-001 Software Configuration Index. These are mock companion records and are not included in this package.