PROJECT SOFTWARE STANDARD

Software Verification Standard

SkyGuard Drone Control Display System (DCDS)

| Field | Controlled value |
| --- | --- |
| Document identifier | DCDS-SVS-001 |
| Revision / status | Revision A / Mock approved baseline |
| Effective date | 04 August 2026 |
| Project baseline | Mock Baseline MB-01 |
| Software level | DAL B (provisional; subject to system safety assessment) |
| Toolchain | Ansys SCADE Suite, SCADE Display and SCADE Test - 2026 R1 project baseline |
| Owner | DCDS Software Engineering Authority |

Purpose: Defines reviews, analyses, requirements-based tests, SCADE Test artefacts, coverage, independence and evidence acceptance for DCDS software verification under DO-178C and DO-331.

Use limitation: Mock standard for training and tailoring; not an approved certification or tool-qualification record. Replace mock approvals, tool versions and system assumptions before live use.

## Approval record

| Role | Mock approver | Disposition | Date |
| --- | --- | --- | --- |
| Software Engineering Authority | A. Engineer | Approved | 04 August 2026 |
| Software Verification Lead | V. Reviewer | Approved | 04 August 2026 |
| Software Quality Assurance | Q. Assessor | Approved | 04 August 2026 |

# 1. Scope and application

This standard applies to the DCDS application software, SCADE models, generated C source, manually developed integration code, configuration data and associated life-cycle data used to present drone state and initiate operator commands. Compliance is mandatory for software placed under the MB-01 certification baseline unless an approved deviation is recorded.

Assurance boundary: The mock baseline assigns DAL B because misleading awareness or loss of command presentation may contribute to a hazardous condition. The command transmission gateway independently validates every request. If that independence is not established, or if an erroneous command could directly cause a catastrophic condition, DAL A objectives and additional independence apply.

## 1.1 System overview

| Panel | Primary content | Safety-related behaviour |
| --- | --- | --- |
| Position (PAN-POS) | Latitude/longitude, map position, altitude, track, groundspeed, home bearing | Reject implausible data; show INVALID or STALE within 0.5 s; never dead-reckon without an explicit DR label |
| Communications (PAN-COM) | C2 link state, signal margin, latency, authentication, active bearer | Annunciate degraded and lost link; distinguish telemetry loss from command-link loss |
| Command and Control (PAN-C2) | Mode, authority, arm state, mission state, pending command, acknowledgement | Two-step guarded commands; inhibit on lost authority, invalid state or stale link; unique transaction ID |
| HUD (PAN-HUD) | Video/sensor view with heading, pitch, roll, altitude, speed and target cue overlay | Remove or flag invalid symbology; prevent stale overlay from appearing current; colour is never the sole cue |

## 1.2 Execution baseline

A deterministic 50 ms application cycle (20 Hz) is the logical execution rate.

Position data is expected at 10 Hz; communications health at 5 Hz; command status and core flight data at 20 Hz.

The video stream may run at 30 Hz but is not safety-credited. SCADE Display symbology is independently refreshed and validity-controlled.

All externally received values carry validity, source time and sequence information. The data-age manager produces CURRENT, DEGRADED, STALE or INVALID status.

Target code is generated as C and integrated with the project platform adaptation layer. Dynamic memory allocation is prohibited after initialization.

# 2. Verification objectives and independence

Verification demonstrates that software requirements, architecture, design, models, generated/manual code, executable behaviour and life-cycle data satisfy their inputs and applicable standards without unintended function. Testing is necessary but does not replace reviews and analyses. Independence is applied according to the provisional DAL B baseline and the approved PSAC/SVP.

| Activity | Minimum performer | Independent approval |
| --- | --- | --- |
| High-level requirements review | Engineer not authoring the item | Verification lead |
| Design/model review | Engineer not authoring the model element | Verification lead for safety-significant logic |
| Test case/procedure review | Verifier independent of requirement implementation | Verification lead |
| Test execution | Qualified test operator or controlled automation | Independent review of results and anomalies |
| Structural/model coverage closure | Verification analyst | Verification lead; SQA confirms process |

SVS-GEN-001  Each verification activity shall identify its input baseline, procedure or analysis method, expected result, actual result, performer, date and review disposition.

SVS-GEN-002  A verifier shall not derive the expected result by copying the implementation expression or model equation under test.

SVS-GEN-003  Pass/fail criteria shall be objective and stated before execution. Observation-only demonstrations are not creditable tests.

SVS-GEN-004  Verification shall address normal range, boundary, robustness, initialization, transition, timing, invalid, stale and failure-recovery behaviour.

SVS-GEN-005  A failed or blocked test shall remain visible. Re-run results shall not overwrite the original result or anomaly linkage.

# 3. Verification methods

| Method | Applied to | Acceptance output |
| --- | --- | --- |
| Review | Requirements, design, models, code, tests, procedures and user-visible wording | Signed checklist, findings and dispositions |
| Analysis | Timing, data/control coupling, stack/memory, traceability, coverage and generator/build output | Reproducible report with thresholds and conclusion |
| SCADE simulation | Suite functional logic and state transitions | Scenario, oracle, results and model baseline |
| SCADE Test execution | Requirements-based model/code tests and campaign management | Controlled test artefacts, logs, pass/fail and coverage |
| Display image comparison | SCADE Display states and symbology | Expected image, inclusion/exclusion masks, tolerance and reviewed diff |
| Target test | Generated/manual object code on representative hardware | Procedure, target configuration, captured outputs and timing |

# 4. Review and analysis standards

SVS-REV-001  Requirements reviews shall assess accuracy, consistency, verifiability, conformity, traceability, algorithmic accuracy, timing, units, ranges and derived requirement status.

SVS-REV-002  Design/model reviews shall assess compatibility with requirements, architecture, data/control flow, initialization, resource bounds, error handling, partitioning and conformance to DCDS-SDS-001 and DCDS-SMS-001.

SVS-REV-003  Code reviews shall confirm traceability, standard compliance, correctness, defensive behaviour and absence of deactivated or extraneous code. Qualified generator credit shall be used only as approved.

SVS-ANA-001  Data coupling analysis shall show that producers, consumers, update rates, validity, units and shared interfaces agree.

SVS-ANA-002  Control coupling analysis shall show that schedule, mode, authority, command state and display navigation invoke functions in the intended conditions and order.

SVS-ANA-003  Worst-case execution time and memory analyses shall cover the release compiler options, target, maximum active alerts, maximum display content and peak input rates.

# 5. Requirements-based test design

Select the requirement and all linked safety constraints; confirm it is testable and approved.

Identify input classes, boundaries, timing, states, invalid cases and required target configuration.

Define an independent oracle with exact values, tolerances, state sequence or approved image.

Implement the case in SCADE Test using named, typed inputs and explicit sampling points.

Review the case and procedure independently before creditable execution.

Execute against the controlled baseline, retain raw results and link every discrepancy to an anomaly report.

| Test ID | Requirement / purpose | Stimulus | Expected result |
| --- | --- | --- | --- |
| TC-POS-001 | HLR-POS-001 nominal position | Valid nav frames at 10 Hz for 5 s | Map symbol and numeric values update within 100 ms; status CURRENT |
| TC-POS-002 | HLR-POS-004 stale position | Stop valid nav frames after steady state | At age >500 ms, remove symbol, dash values and show POSITION STALE |
| TC-COM-004 | HLR-COM-006 command-link loss | No command-link update for 1501 ms; telemetry remains valid | COMMAND LINK LOST warning; telemetry status remains independently valid |
| TC-C2-006 | HLR-C2-011 guarded command | Select RETURN HOME; confirm within 5 s with valid authority | Exactly one request with non-zero unique ID; state SENT |
| TC-C2-007 | No duplicate command | Hold/double-activate confirm; replay old ack | One send only; mismatched/old ack does not produce ACCEPTED |
| TC-HUD-005 | HLR-HUD-009 invalid attitude | Attitude age advances through 249, 250 and 251 ms | Boundary behaviour matches requirement; invalid cue shown after threshold |
| TC-SYS-010 | Safe restart | Restart in each C2 transaction state | No command emitted; all external data INVALID until refreshed |

# 6. SCADE Test artefact standard

| Artefact | Naming | Mandatory content |
| --- | --- | --- |
| Test project | DCDS_TEST_<release> | Tool/version, model/executable binding, target configuration and campaign definition |
| Test case | TC-<panel>-<nnn> | Requirement links, purpose, preconditions, stimuli, expected results, tolerances and cleanup |
| Procedure | TP-<level>-<nnn> | Environment setup, execution commands, sampling, evidence capture and restoration |
| Expected image | IMG-<panel>-<test>-<state> | Reference resolution/theme/font pack, inclusion/exclusion mask and pixel tolerance |
| Result | TR-<release>-<campaign>-<run> | Baseline, time, operator/automation ID, pass/fail, raw log, screenshots and anomalies |

SVS-TST-001  SCADE Test cases shall not contain unreviewed calculations that duplicate the implementation. Reusable oracle functions require their own requirements, review and verification.

SVS-TST-002  Scenario time shall be explicit. A test shall not rely on workstation speed, animation timing or manual reaction time for a pass criterion.

SVS-TST-003  Image comparison shall constrain the relevant region and tolerance. Broad masks, unconstrained anti-aliasing or whole-screen percentage thresholds require justification.

SVS-TST-004  Test data shall identify engineering units and include values immediately below, at and immediately above every safety-significant boundary.

SVS-TST-005  Tool automation that can fail to detect an error shall be qualified as determined by the Tool Qualification Plan or its output shall be independently checked.

# 7. Coverage and completeness

| Coverage | DAL B mock objective | Closure action |
| --- | --- | --- |
| Requirements coverage | Every approved requirement has one or more passing verification activities | Add case/analysis or justify non-test method |
| Model coverage | Exercise model decisions and state transitions appropriate to model classification; analyse uncovered elements | Add requirement-based test, identify deactivated logic or remove unintended function |
| Source structural coverage | Statement and decision coverage; MC/DC where required by applicable objective or project safety allocation | Run code coverage; analyse generated/manual source gaps |
| Object code | Additional verification where compiler-generated object cannot be directly traced to source or qualified generation evidence | Disassembly/object analysis and targeted tests |
| Data/control coupling | All integration couplings exercised and analysed | Add integration scenario or correct interface |

Coverage rule: Coverage is a completeness check, not a source of requirements. Testing added solely to cover implementation structure shall first determine whether the structure implements a missing requirement, deactivated code or unintended function.

# 8. Display and human-interface verification

SVS-HMI-001  Every panel shall be verified in day/night themes at the approved resolution for normal, degraded, stale, invalid, alert, overlap and maximum-content states.

SVS-HMI-002  Text shall be checked for wording, unit, truncation, contrast and localisation bounds. Safety text shall not be clipped or replaced by an icon alone.

SVS-HMI-003  Interactive controls shall be tested at target input boundaries, including rapid repeat, simultaneous events, drag-off/cancel and disabled-state activation.

SVS-HMI-004  HUD verification shall separately stimulate video loss, frozen video, overlay loss and stale flight data to demonstrate layer independence.

SVS-HMI-005  A representative human-in-the-loop evaluation may supplement but shall not replace deterministic software verification evidence.

# 9. Target integration and regression

Run representative-hardware tests for scheduling, I/O conversion, graphics startup, frame timing, watchdog, restart and maximum-load behaviour.

Confirm generated code and the platform adaptation layer use the same type sizes, byte order, units and compiler ABI.

A change-impact analysis selects regression tests from changed requirements/models/code, interfaces, safety dependencies and previously failed areas.

The minimum release regression set includes all command gating, link loss, stale-data, restart and alert-priority cases.

# 10. Anomaly and result control

| Severity | Definition | Release treatment |
| --- | --- | --- |
| 1 - Safety | Could mask or create safety-significant behaviour, invalidate evidence or cause unintended command | Release blocked until corrected and independently reverified |
| 2 - Major | Incorrect requirement behaviour or repeatable functional failure without immediate safety effect | Release blocked unless formally accepted with safety assessment |
| 3 - Minor | Non-safety presentation or documentation defect | May be deferred with approved impact and regression commitment |
| 4 - Observation | Improvement or nonconformance with no product effect | Disposition before final certification data closure |

# Appendix A - Verification completion checklist

| Completion criterion | Objective evidence | Status |
| --- | --- | --- |
| All applicable standards reviews complete | Review records and closed findings | Open / Complete |
| Requirements and traceability complete | Trace report with no unexplained gaps | Open / Complete |
| All planned SCADE Test campaigns passed | Signed campaign summaries and raw results | Open / Complete |
| Coverage achieved and gaps dispositioned | Model/source/object coverage analysis | Open / Complete |
| Target integration and timing passed | Target logs, WCET/memory and watchdog results | Open / Complete |
| Anomalies assessed and release baseline identified | Problem reports, SCI and SQA audit | Open / Complete |

## Conformance and deviations

A work product conforms when every applicable shall statement is met and the review evidence is recorded. A deviation shall identify the affected rule, technical rationale, safety impact, compensating verification, owner and approval. Software Quality Assurance records deviations and confirms closure before release.

# References

| Reference | Use in this standard |
| --- | --- |
| RTCA DO-178C | Software Considerations in Airborne Systems and Equipment Certification, 13 December 2011. |
| RTCA DO-330 | Software Tool Qualification Considerations, 13 December 2011. |
| RTCA DO-331 | Model-Based Development and Verification Supplement to DO-178C and DO-278A, 13 December 2011. |
| FAA AC 20-115D | Airborne Software Development Assurance Using EUROCAE ED-12() and RTCA DO-178(). |
| FAA Order 8110.49A | Software Approval Guidelines. |
| Ansys SCADE Suite | Official product information for model-based development and qualifiable code generation. |
| Ansys SCADE Display | Official product information for safety-critical HMI development and code generation. |

FAA AC 20-115D | FAA Order 8110.49A | Ansys SCADE Suite | Ansys SCADE Display

Project references: DCDS-PSAC-001 Plan for Software Aspects of Certification; DCDS-SDP-001 Software Development Plan; DCDS-SVP-001 Software Verification Plan; DCDS-SCMP-001 Software Configuration Management Plan; DCDS-SQAP-001 Software Quality Assurance Plan; DCDS-SCI-001 Software Configuration Index. These are mock companion records and are not included in this package.